Due to many requests to e-mail my 550 performance / power charts by members I started a new post with the appropriate title “550 Power charts”.

First let me put a disclaimer: These charts are for reference only and should NOT be used as a substitute for the POH.

As there were no good power charts available with the engine conversion, I have tried to combine information received from VK, TCM and myself into a comprehensive chart that can be used to select desired power settings or % power setting ROP. I have combined the information into two PDF files for easy viewing and have included the excel sheet for adaptation by he  individual pilot for his / her airplane.

The excel power sheet should only be amended by each individual user to reflect their actual Fuel flows in the legend for either ROP & LOP given their desired temperature limits. I set my parameters to achieve these fuel flows at max 380 CHT & 1650 TIT.

Fuel Flows will vary with each individual engine so the end user must establish what his/her desired limits are and measure the fuel flows for their Eigen. Note these fuel flow will change with season due to outside temperature , climb speed and will also vary slightly with altitude but they give you a target to aim for.

Now to the attached carts ( Chart.pdf will provide more detail for the end user)

Note the Hi / Lo fuel flow. Hi is ROP and Lo is LOP for my engine you need to determine where your individual fuel flows in respect to your temperature which you are comfortable with ( Temps are key I use max 380 Cyl and 1650 TIT) are and amend the legend figures accordingly. Do not change the graph as that is the power chart from TCM. The Power charts assumes ROP for the settings. When you lean you will still have the same power settings but you will not make the same HP  ( see TCM-pwr chart) on the lean side like on the rich side. Your speeds will drop between 7-10kts on the lean side for the shown power setting and therefore the speed chart of the old 520 reflects how the 550 will perform LOP (i.e. 520 ROP ≈ 550 LOP)

Attached are the excel charts as well as in pdf format and the power chart to depict the difference between ROP and LOP

Note that the fuel flows are for my plane and you must adjust correct them for your airplane as they vary from plane to plane for the respective power settings.

I generated the 550 performance chart in Excel as a comparison to the 520. The reason for the two 75% power line is because the first three lines (55%, 65%, 75%) are for the IO 520 and the second 75% as well as the MCP line is for the IO 550. From that you can derive that the IO 550 is about 10-14 kts faster than the IO 520 at respective power settings.

Look at it this way

IO 520              IO 550

45%      =          40%

55%      =          49%

65%      =          58%

70%      =          62%

75%      =          66%

85%      =          75%

So if you want to change the legend than use the above shown values

Andre